Elcus Electronic Company (http://www.elcus.ru)

TА1-PCI

# PCI MIL-STD-1553

# (GOST R 52070-2003) interface module

Interface module is implemented as a standard universal 3.3V/5V PCI card with the dimensions 107mm х 175mm. The base address of the input/output ports and the interrupt line on the PCI bus are programmable.

Interface module TА1-PCI is designed to connect the IBM PC/AT with the PCI bus to the MIL-STD-1553B (national standard GOST R 52070-2003) dual redundant bus. Terminal operation mode (bus controller (BC), remote terminal (RT), bus monitor (MT)) is set by program.

The device contains the redundant transceiver, dual-port 64Кх16 RAM, PCI interface controller, protocol IC, implementing the control over the modes required. In the address space of the input/output ports the device takes 32 subsequent addresses and one interrupt line.

The device meets the requirements of the RT Validation Test Plan.

Internal interrupt FIFO capacity is 256 words.

There is a 32 bit programmable timer for message time tagging.

There are three main monitor operation modes – message monitor (MTM), words monitor (MTW) and combined monitor. All monitor modes allow setting the RT address so that the device can be used as an addressed monitor. In the addressed message monitor mode the device responds to the commands sent to it as an RT and receives messages upon the predefined list of addresses. When operating in the combined monitor mode, while the incoming data is recognized as a message this data is recorded by the message monitor. In parallel the word monitor records any transmitted word if it starts with the sync signal and two valid bits.

In the RT mode the received messages can be buffered for each of the subaddresses. Message receiving timer can be programmed upon BC commands. The message receiving/transmitting can be disabled for the specified subaddresses.

In the BC mode the device allows automatic chain transmitting. There is a programmable   message error processing with the possibility of automatic retry and switching the bus number. Status words masking is implemented. Status word (STATUS) timeout is programmable.

The possibility of transceivers and bus status testing is implemented.

1. Components and main characteristics

Table 1. Main characteristics

| Parameter | Units | min | typ | max |
| --- | --- | --- | --- | --- |
| Receiver Differential input voltage | Vp-p | 0.65 |  | 40 |
| Transmitter Differential output voltage measured in the bus channel Signal rise/fall time | Vp-p ns | 6 100 | 6.5 150 | 300 |
| Power Supply Requirements +5V 12V +12V * pause * 25% transmission time * 50% transmission time * 100% transmission time | mA mA mA mA mA mA | 5 25 45 85 | 60 20 65 110 200 | 200 50 100 150 250 |
| Time Parameters - Delay between the start of the BC and the beginning of the transmission - BC/MT/RT STATUS timeout (programmable) - RT response time - Interrupt delay at the end of the message - Controlled bus jamming | s s s s s | 3 14.5 760 | 4.5 | 63.5 6 |
| Temperature Range - Operation - Storage The product can be supplied with the extended temperature range | C C | +5 -25 |  | +40 +85 |

Data bus redundancy is provided at the transceiver level. The device can be connected to the bus using direct coupled or transformer coupled connection. Onboard termination resistors 75 Ohm can be used if the device is connected directly at the end of the bus.

IC АРА150-PQ208 contains two decoders and one coder of the Manchester code, protocol controllers of the BC, RT and MT modes, addressing scheme and the dual-port RAM arbiter.

IC PCI9030 implements the functions of the PCI bus interface controller.

Dual-port RAM 64Kх16 is used to store data and control information.

The device includes the 48 MHz clock generator.

The device contains the voltage converter for IC АРА150-PQ208, PCI9030 and RAM, converting voltage  from +5V to +3.3V and +2.5V.

TА1-PCI main characteristics are shown in table 1.

TА1-PCI module uses TA device program model.

TA device program model description is in TA_eng.DOC.

Jumpers and connectors

Fig. 1 shows the main jumper fields and device connectors.

Fig. 1. Device drawing.

Connector X2 – MIL-STD-1553B bus A

Connector X3 – MIL-STD-1553B bus B

Jumper JP1 – connecting the termination resistor (75 Ohm) of the bus A

Jumper JP2 – connecting the termination resistor (75 Ohm) of the bus B

MIL-STD-1553B Bus interface

TА1-PCI device is connected to the bus channel using two DB09 connectors. Fig. 2 shows the connection drawing. Contacts 6 and 9 are used for direct coupled connection. Contacts 7 and 8 are used for transformer coupled connection. 75 Ohm resistors can be connected with jumpers JP1, JP2 (not shown on the drawing).

Fig 2. Connecting the device to data bus.